STAR-NOSE CAPACITIVE SENSOR

Technical Report: System Status, ML Roadmap & Signal Quality

May 2026

Jonathan Tirado

Figure 1. The 19-cell hexagonal capacitive array with physical coordinates (mm).

# 1. Current System Overview

## 1.1 Hardware

The sensor is a capacitive tactile array — the Star-Nose Sensor. Of the 252 raw cells in the skin, 19 are actively used, arranged in a hexagonal diamond grid spanning approximately 38 mm in diameter. Each cell outputs a capacitance value that is read over a serial connection (USB/UART) at 115,200 baud by a host computer.

The sensor is mounted on a UR5 6-DOF robot arm, which provides millimetre-accurate ground-truth labels during data collection: TCP position (x, y, z in mm) and force/torque readings (Fx, Fy, Fz, Tx, Ty, Tz). This combination eliminates the need for manual annotation.

Figure 2. Full system architecture from hardware to live inference.

| Parameter | Value / Description |
| --- | --- |
| Sensor cells | 19 active (from 252-cell Star-Nose skin) |
| Cell spacing | 8 mm (column), 7 mm (row) |
| Sensor diameter | ~38 mm (outer boundary) |
| Serial rate | 115,200 baud — USB UART to host PC |
| Frame rate | ~16 Hz (observed from session logs) |
| Robot arm | Universal Robots UR5 (6-DOF, ±0.1 mm TCP accuracy) |
| Force sensor | Integrated F/T at TCP (Fx, Fy, Fz, Tx, Ty, Tz) |
| Normalization | value = max(0, min((raw - baseline) / SENSITIVITY, 1))^GAMMA |
| Hyperparameters | SENSITIVITY = 30.0, GAMMA = 0.5 |

## 1.2 Software Stack

The software is organized into a live pipeline and an offline pipeline.

sensor.py — Serial reader running in a background thread. Handles auto-reconnection, calibration (single-frame baseline), normalization, and writes the current state to a shared JSON file at 30 Hz for inter-process access.

data_logger.py — Synchronizes UR5 TCP/F/T data with sensor readings, logs all channels to timestamped CSV files. Labels each frame with ur5_point, ur5_pressing, and ur5_done flags.

build_dataset.py — Post-processes CSV logs into ML-ready datasets: frames.csv (one row per frame during pressing) and events.csv (one row per complete press event, with peak and mean aggregations). Computes spatial features: centroid (x, y), spread, n_active, and diameter estimate.

train.py — Trains four independent sklearn pipelines (StandardScaler + model). Supports Random Forest, SVM, and MLP backends via --model flag. Saves .pkl files to saved_models/.

predict.py — Loads all saved models and runs live inference on the sensor stream, outputting contact class, (x, y) position, depth, and diameter simultaneously.

## 1.3 Current ML Tasks

Four regression/classification tasks are currently trained and deployed:

| Task | Type | Target | Current Performance |
| --- | --- | --- | --- |
| Contact classifier | Classification | Which of 19 points (1–19) | High accuracy on trained points; no interpolation |
| Position regressor | Regression (2D) | (x, y) in mm | MAE reported per session; limited between-point generalization |
| Depth regressor | Regression (1D) | z indentation in mm | MAE ~0.3–0.5 mm on trained range; depends on depth coverage |
| Diameter estimator | Regression (1D) | Contact footprint (mm) | Derived from pressure spread; approximate |

## 1.4 Dataset Status

As of May 2026, the labeled dataset contains approximately 28,000 frames across multiple sessions collected with the UR5. Sessions cover the canonical 19-point grid at varying depths. Several sessions use an empty dome (no object pressed), solid dome, and curved surface configurations.

| Session group | Sessions | Labeled frames | Notes |
| --- | --- | --- | --- |
| dome_empty_tuesday (×6) | 6 | ~8,700 | Empty dome, 19-pt grid |
| dome_empty_burcu | 1 | ~1,400 | Different operator |
| test_curved (×2) | 2 | ~2,900 | Curved surface |
| solid_domes | 1 | ~1,400 | Solid dome object |
| ursim_19pts (latest) | 1 | ~1,795 | URSim — 19 pts, clean |
| Total (approx.) |  | ~28,000 frames |  |

# 2. ML Learning Roadmap

The current pipeline handles static contact detection well. The next phase extends it to three new capabilities: rotation estimation, shape recognition, and displacement/trajectory tracking. This section describes the recommended approach for each.

Figure 3. Current ML pipeline (left) vs. proposed multi-task pipeline with extended features (right).

## 2.1 Feature Engineering Extensions

The current feature vector (19 cell values + 4 spatial scalars = 23 features) is sufficient for static position and depth. For rotation and shape, second-order spatial moments must be added. These are computed from the weighted positions of active cells, where weights are the normalized cell values:

| Feature | Formula | Physical meaning |
| --- | --- | --- |
| Ixx | sum(v_i * (y_i - cy)^2) | Spread of activation along Y axis |
| Iyy | sum(v_i * (x_i - cx)^2) | Spread of activation along X axis |
| Ixy | sum(v_i * (x_i - cx)(y_i - cy)) | Cross-moment — captures tilt/rotation |
| theta | 0.5 * atan2(2*Ixy, Iyy - Ixx) | Principal axis angle of pressure ellipse |
| eccentricity | (L1 - L2) / (L1 + L2) | Ratio of eigenvalues — sphere=0, edge~1 |
| delta_cx, delta_cy | centroid(t) - centroid(t-1) | Centroid velocity for displacement |
| delta_spread | spread(t) - spread(t-1) | Change in contact area |

Adding these 7 features brings the total feature vector to 30 dimensions, which all existing model types (RF, SVM, MLP) can handle without architectural changes.

## 2.2 Rotation Estimation

Rotation estimation predicts the orientation angle of an elongated object (cylinder, edge, flat stamp) pressing on the sensor. The key challenge is that angles are periodic: 180° and 0° are equivalent for most symmetric probes.

Recommended approach: predict the 2D vector (cos(2theta), sin(2theta)) as a regression target instead of the raw angle. This is smooth, continuous, and handles wrap-around automatically. Recover the angle with atan2(sin_output, cos_output) / 2. A multi-output MLPRegressor with hidden layers (256, 128) is sufficient for this task with the moment features above.

Expected data requirement: 8 angles x 10 positions x 3 depths x 5 reps = 1,200 labeled events (one lab session with the UR5).

## 2.3 Shape Recognition

Shape recognition classifies what type of object is pressing on the sensor. With 4-7 distinct shape classes, this is a tractable classification problem given the moment features.

| Shape | Key signature | Distinguishing features |
| --- | --- | --- |
| Sphere | Circular footprint | eccentricity ~ 0, spread proportional to depth |
| Flat disk | Large uniform area | high n_active, low spread variance, eccentricity ~ 0 |
| Cylinder (normal) | Elliptical ring | eccentricity > 0.5, theta stable with depth |
| Edge / rectangle | Line activation | eccentricity ~ 0.8-1.0, very low Iyy or Ixx |
| Ring / donut | Annular pattern | centroid activation low, peripheral high |

A RandomForestClassifier with 300 trees on the 30-feature vector is the recommended starting point. If more than 7 classes are needed, a small MLP (128, 64) with softmax output will generalize better.

## 2.4 Displacement & Trajectory Tracking

Displacement tracking estimates how the contact moves across the sensor surface over time (sliding, dragging). Two levels of complexity are possible:

- Instantaneous velocity: add delta_cx and delta_cy to the feature vector and use the existing static regressors. This requires no architectural change and works at 16 Hz.

- Full trajectory reconstruction: use an LSTM over a sliding window of 10 frames (input shape: [batch, 10, 30]). The LSTM output predicts (x, y) at each timestep. This is the correct approach for continuous tracking but requires sequence-format training data from the UR5 slide trajectories.

For the LSTM, PyTorch Lightning is recommended as a lightweight extension to the existing Python codebase. A 2-layer LSTM with hidden size 64 and a linear output head is sufficient for the sensor resolution.

# 3. Data Collection Protocol

Figure 4. Three data collection protocols for the UR5-sensor system.

## 3.1 Protocol A — Static Presses (Position, Depth, Diameter)

This protocol is already partially implemented. The recommended extension is to increase depth coverage and add repeated presses for variance estimation.

| Parameter | Specification |
| --- | --- |
| Grid points | 19 canonical + 6 intermediate (between adjacent pairs) |
| Depth levels | 0.5, 1.0, 1.5, 2.0, 3.0 mm (5 levels) |
| Repetitions | 5 presses per configuration, 1.5 s dwell time |
| Approach speed | 3 mm/s (slow enough for sensor settling) |
| Total events | 25 pts x 5 depths x 5 reps = 625 labeled events |
| Calib. check | Press point 10 (center) at 1.5 mm at start and end of session |

## 3.2 Protocol B — Rotation Presses

Mount an asymmetric probe (cylinder or flat rectangular stamp, 8-12 mm long) on the UR5 TCP. Vary the TCP roll angle in 8 steps of 22.5° from 0° to 157.5°.

| Parameter | Specification |
| --- | --- |
| Probe geometry | Cylinder (8 mm length, 3 mm diameter) or flat stamp (10 x 3 mm) |
| Angles | 0, 22.5, 45, 67.5, 90, 112.5, 135, 157.5 degrees (8 steps) |
| Grid points | Subset of 10 representative positions (center + ring) |
| Depth levels | 1.0 and 2.0 mm (2 levels) |
| Repetitions | 3 presses per configuration |
| Ground truth | TCP orientation (rx, ry, rz) logged — angle computed from rz |
| Total events | 8 angles x 10 pts x 2 depths x 3 reps = 480 labeled events |

## 3.3 Protocol C — Shape & Sliding

Two sub-protocols: (a) discrete presses with different probe shapes, (b) continuous linear trajectories for displacement training.

- Shape presses: 4 probe geometries (sphere, flat disk, edge, ring), 10 positions each, 2 depths, 3 reps = 240 labeled events.

- Slide trajectories: 6 linear paths across the sensor (3 horizontal, 3 diagonal), constant depth 1.5 mm, speed 2 mm/s. Each path generates ~100 labeled frames.

- Label: shape_class (integer 0–3) and trajectory_id for sliding frames.

## 3.4 General Best Practices

- Always let the robot pause 0.5 s before logging the first pressing frame (settling time).

- Clean the sensor surface with isopropyl alcohol between shape changes.

- Add a calibration check press (center, 1.5 mm) at session start and end to detect drift.

- Log ambient temperature — capacitive sensors drift with temperature gradients.

- Mark sessions with the probe type in the filename (e.g., sphere_session_YYYYMMDD.csv).

# 4. Signal Quality Issues & Proposed Fixes

Analysis of the ursim_19pts_session_20260525 log reveals three distinct noise sources that degrade both live inference and training data quality. All three require fixes before extending the ML pipeline.

Figure 5. Noise characterization: idle baseline (top-left), SNR (top-center), transition smear (top-right), spike distribution (bottom-left), and filter effect (bottom-right).

## 4.1 Noise 1 — High Idle Baseline

| Observed | Mean idle cell value ~0.32 across all 19 cells (should be ~0). SNR only 3.5x. All cells show idle std of ~0.22. |
| --- | --- |
| Root cause | Calibration is captured from a single frame immediately after connection. If the electronics have not settled, or if the UR5 arm is nearby generating EMI, the baseline is wrong. The GAMMA=0.5 exponent then amplifies small errors — a raw offset of 10 counts becomes (10/30)^0.5 = 0.58 instead of 0.33. |
| Fix | 1. Multi-frame calibration: accumulate N_CALIB=20 frames and use their mean as the baseline instead of a single frame. Estimated improvement: baseline noise reduced by 1/sqrt(20) ~ 78%. 2. Dead-zone clamp: after normalization, set values below DEAD_ZONE=0.12 to zero and rescale: v = max(0, v - DEAD_ZONE) / (1 - DEAD_ZONE). This eliminates idle chatter without affecting the signal region. 3. Periodic recalibration: expose a recalibrate() call triggered every N minutes or when no contact is detected for >10 s. |

## 4.2 Noise 2 — Transition Smear

| Observed | At every press start and end, cell values take ~3 frames (~190 ms at 16 Hz) to settle. These frames are currently included in the training dataset as valid pressing samples, teaching the model incorrect patterns. |
| --- | --- |
| Root cause | Two combined effects: (a) mechanical settling — the sensor membrane takes ~100 ms to reach equilibrium after first contact; (b) sensor RC time constant — capacitive sensors have an inherent low-pass response determined by the measurement circuit. |
| Fix | 1. Dataset: in build_dataset.py, add a MARGIN_FRAMES=3 parameter that skips the first and last 3 frames of each press event before emitting rows to frames.csv. 2. Live pipeline: in the UR5 collection routine, add a 0.5 s wait after the ur5_pressing flag goes high before starting to log sensor frames. This gives the sensor time to settle before any frames are recorded. |

## 4.3 Noise 3 — Frame-to-Frame Spikes

| Observed | 1.2% of pressing frames contain at least one cell with an impossible frame-to-frame jump (¦delta¦ > 0.12 normalized units, >3x global std). These are physically impossible at 16 Hz given the sensor mechanics. |
| --- | --- |
| Root cause | Serial framing artifacts: partial UART lines received when the host reads mid-transmission, or USB interrupt collisions under load. The parser in sensor.py checks for len(parts) != SKIN_CELLS but does not validate value continuity. |
| Fix | 1. Live pipeline (sensor.py): after parsing a new frame, compute the max absolute delta from the previous frame. If any cell exceeds SPIKE_THRESHOLD=0.25 normalized units, discard the frame and retain the previous values. 2. Dataset (build_dataset.py): add a spike-rejection pass that drops rows where any cell delta exceeds the threshold before writing to frames.csv. 3. Smoothing (sensor.py): apply a per-cell EMA with alpha=0.35 after spike rejection. This attenuates residual high-frequency noise while keeping latency low (~2 frame lag = ~125 ms). |

## 4.4 Summary of Proposed Signal Processing Chain

The complete filter chain applied to each incoming frame, in order:

| Step | Operation | Implementation |
| --- | --- | --- |
| 1 | Multi-frame calibration | Average N_CALIB=20 frames for baseline; re-run every 10 min idle |
| 2 | Standard normalization | (raw - baseline) / SENSITIVITY, clipped to [0, 1] |
| 3 | Gamma compression | value^GAMMA (GAMMA=0.5) — keep for perceptual scaling |
| 4 | Spike rejection | Discard frame if any ¦delta_cell¦ > SPIKE_THRESHOLD (0.25) |
| 5 | Dead-zone clamp | v = max(0, v - DEAD_ZONE) / (1 - DEAD_ZONE), DEAD_ZONE=0.12 |
| 6 | Per-cell EMA | v_filtered = alpha * v_new + (1-alpha) * v_prev, alpha=0.35 |

# 5. Next Steps

| # | Task | Details | Priority |
| --- | --- | --- | --- |
| 1 | Fix signal pipeline (sensor.py) | Multi-frame calib, dead-zone, EMA, spike rejection — new branch recommended | HIGH |
| 2 | Fix dataset builder (build_dataset.py) | Add MARGIN_FRAMES=3, spike rejection pass, moment features (Ixx, Iyy, Ixy, theta, eccentricity) | HIGH |
| 3 | Collect rotation data (Protocol B) | One UR5 session, asymmetric probe, 8 angles x 10 pts x 2 depths | MEDIUM |
| 4 | Collect shape data (Protocol C) | One UR5 session, 4 probe shapes + slide trajectories | MEDIUM |
| 5 | Train rotation regressor | (cos2theta, sin2theta) targets, MLP(256,128), moment features | MEDIUM |
| 6 | Train shape classifier | RF(300 trees), 30-feature vector including moments | MEDIUM |
| 7 | Multi-task MLP (optional) | Shared (256,128) trunk, 6 output heads — better generalization | LOW |
| 8 | LSTM for trajectory (optional) | PyTorch, 2-layer LSTM hidden=64, 10-frame window | LOW |

End of Report